Measure, Monitor, Act, Display

About the Project

This project is a simple monitoring and control system built using an Arduino UNO. The main idea is to read sensor values, check if they reach a certain limit, and then take an action.

An MQ-2 gas sensor is used to monitor the gas level. When the gas level reaches the set limit, the Arduino turns on the buzzer and DC motor. The system can also be controlled manually using a MATLAB GUI.

How It Works

The Arduino reads the gas sensor through its analog input. The sensor reading is converted into a digital value using the Arduino's built-in ADC.

If the gas reading is higher than the selected limit, the buzzer and motor are turned on automatically.

The Arduino is also connected to MATLAB through a USB serial connection. MATLAB receives the gas readings and allows the user to manually activate or deactivate the system.

In simple terms:

MQ-2 Sensor → Arduino → Check Gas Level → Buzzer + Motor

↑

MATLAB GUI

Components Used

- Arduino UNO

- MQ-2 Gas Sensor

- Buzzer

- DC Motor

- Resistors

- Voltage Divider

- USB Cable

Pin Connections

| Component | Arduino Pin |
| --- | --- |
| MQ-2 Gas Sensor | A0 |
| Buzzer | D2 |
| DC Motor | D3 |

The Arduino communicates with MATLAB through the USB serial connection.

Arduino Code

The Arduino reads the gas sensor every 200 ms and sends the reading to MATLAB using serial communication.

The serial communication speed is 9600 baud.

MATLAB can also send commands to the Arduino:

- 1 → Turn the system ON manually

- 0 → Turn the system OFF manually

The automatic gas detection still works even when MATLAB is connected.

MATLAB GUI

The MATLAB App Designer GUI is used to display the gas level and the system status. It also includes a button that allows the user to activate the system manually.

The GUI communicates with the Arduino through the serial port.

Tinkercad Simulation

Before connecting the real Arduino, the circuit was tested in Tinkercad. The simulation was used to check the MQ-2 gas sensor reading and the automatic buzzer response.

Important Note

For measuring an input voltage higher than the Arduino's allowed analog input range, a voltage divider is required before connecting the signal to the Arduino. This protects the Arduino and allows the voltage to be measured safely.

Project Structure

Project

Research

Simulation

Arduino

MATLAB

README.md

Conclusion

The project combines sensing, processing, control, and communication in one system. The Arduino handles the sensor readings and output devices, while MATLAB provides a simple way to monitor the system and control it manually.